Serial Fix 1992

by Medical Designs Software

Version 1.1

Written by: Bill Penner

Icons created by: Bruce D. Noonan, M.D.

This program may be distributed and used, free of charge,

provided that the program is distributed in complete form and

UNmodified.  This software is not to be sold, however, may be

included  on  disks which are sold for a small handling fee.

Medical  Design  Software  retains  all  copyright  exclusive

rights except exclusive distribution.  This software consists

of  the  following  files:  SERIALFX.PRG,  SERIALFX.CPX,  and

SERIALFX.DOC (this file).

PURPOSE

Serial  Fix was written as an upgrade to the old TurboCTS

program,  written  by Bill Penner, long ago, as an attempt to

fix  the old RTS/CTS problem.  The problem the ST/STe/TT has,

occurs  when  a  high speed modem is used in conjunction with

ZMODEM  or  any near-continuous flow transfer program is used

to  send  data.   The ST/TT has problems properly recognizing

when  the  modem  deasserts  CTS  (telling the ST/TT that the

buffer  is  full  and don't send any more data), resulting in

the  ST  overflowing the modem's buffer.  This results in bad

data  being  sent  which  results  in CRC errors, packet size

problems, and other data transmission errors.

TurboCTS was only a partial fix of the problem.  TurboCTS

did manage to speed things up to the point where CTS response

was  generally  ok, however, that program did not work on the

TT,  did  not support XBRA programming requirements, and only

worked for RTS/CTS handshaking.

Serial  FIX  is  a replacement for TurboCTS and will work

with  all  versions  of  TOS  (1.0 through 3.06).  Serial FIX

supports  both  RTS/CTS, XON/XOFF, and no handshaking modes.

The  desired  handshaking  mode is set from the RS232 control

panel  or other program as you currently do now.  The new fix

is  a  REAL fix to the RTS/CTS problem.  Previous fixes (like

TurboCTS) only fixed the symptoms of the problem.

INSTALLATION:

Serial  FIX  is  composed of two parts, a program for the

AUTO  folder  and  a  CPX file to be used with the extensible

CONTROL (XCONTROL.ACC) panel.

SERIALFX.PRG  is  the  heart of the Serial FIX solution.

SERIALFX.PRG is intended to be installed in your AUTO folder,

although  it  can  be  ran from the desktop, and replaces the

current  serial  routines for the ST compatible port.  The ST

compatible  port is the only port on the ST and STe machines,

or the Modem 1 port on the MegaSTe and TT machines.

SERIALFX.CPX  is  a  control  panel  to  be used with the

extensible  control panel (XCONTROL.ACC).  The CPX is used to

lock  the  serial  routines  into  using RTS/CTS handshaking,

regardless   of   the   desires   of  a  program.   The  file

SERIALFX.CPX  is  to be placed in in your folder of CPX files

(often called 'CPX').

OPERATION:

SERIALFX.PRG:   Once the program has been started (either

booted  in  the  'AUTO'  folder  or run from the desktop), no

special  operations  are  needed.   The  program replaces the

system  routines  and  the port speed and handshaking are set

just  as  they  are now, either by terminal software, CONTROL

panel, or the extensible control panel.

SERIALFX.CPX:   This  program  will allow you to lock the

handshaking  in RTS/CTS mode.  This is often needed because a

number  of  programs have been written to ignore RTS/CTS mode

(like FLASH).  If the CPX is not used to lock the mode on, if

the  mode  is  set  from  the  control  panel, and a terminal

program  is  started, very likely, the program would turn off

RTS/CTS.  With the mode locked, the program will be unable to

disable  RTS/CTS.   The  CPX will give you the option to save

the  current lock setting.  Saving the setting will cause the

selected mode to be entered every time the CPX boots up.  So,

note,  to  be  able to lock the RTS/CTS mode on, you must use

both the SERIALFX.PRG and the SERIALFX.CPX.

PROGRAMMING INFORMATION:

This  version  of  the  software should now be fully XBRA

compliant.    When  the  software  starts  and  installs  the

replacement routines, the TOS version is checked to determine

the  best  method  of installation (read for maximum speed).

Under  TOS  1.0,  traps  1,  13,  and  14 are intercepted and

processed.   Under  TOS  1.2,  1.4,  and  1.6  (non-Bconmap()

compatible  machines  after  1.2),  the  xcon vector table is

used,  and  only  trap 14 is used to receive Rsconf() calls.

Under  Bconmap()  compatiable  systems,  bconmap (port 6) and

xcon  vectors  are  modified  so that no trap interception is

necessary.

Other  software  can be used to lock or unlock the serial

routines.   To  perform this function, look in the cookie jar

for  a 'MDSa' cookie.  The long data byte is a pointer to the

following data structure:

struct {

int   lockfg;        /* RTS/CTS locked flag (bit 1 =

locked) */

int   (*setlock)()    /*  Routine to set lock status

*/

} serfx;

The  lock  status is set by pushing an integer with the value

of  either  0  (to  disable  lock)  or 1 (to enable) and then

jsr'ing  the  setlock  routine  (i.e. (*serfx->setlock)(1) to

enable  the  lock).   The  call  can be made in user or super

mode.  That is all there is to it.

WRAP UP

Well, ATARI has indicated that this problem (the real one

that  has  been haunting us for many years) is finally fixed.

The  fix  should  show up in some future release of ROMs from

ATARI.   Until  that  time,  this  software  should  fix  the

problem.

As  normal,  PLEASE  let me know if you have any problems

with  the  program.   I have attempted to make the program as

compatible with other software as possible.

Bill Penner, IAAD Member

3235 Wright Avenue

Bremerton, WA  98310-4753

Phone: (206) 373-4840 (before 9pm Pacific)

GEnie: BPENNER

CHANGES SINCE VERSION 1.0

Two  bugs  that  were  discovered  were fixed.  Both bugs

would  only  occur  if  no  cookie  jar  was  found.  One bug

resulted  in  creating  a cookie jar that was full of garbage

and  the  other would not build a cookie jar.  The second bug

was  the more common, which would result in the CPX reporting

that  the  program  was not loaded.  Running the SERIALFX.PRG

program a second time often worked to fix the problem.