SERPTCH2.PRG DOCUMENTATION April 15, 1991 by Allan Pratt, Atari Corp.

All versions of Mega STe TOS and TT TOS up to 2.05/3.05 have code in the

BIOS to communicate using the MFP and SCC serial ports.  They all share a

few problems:

1.	If an SCC port's word size is set to less than 8 bits, the high bit

of the byte you read is not correct.  If you are using parity, the

high bit is the parity bit as received.  If you are not using

parity, it's a one.  It should be masked off (read as zero), as it

is when you use the serial port on an ST.

2.	If you use parity on an SCC port, and there is a parity error, the

character will be placed in the input buffer anyway.  This is not

the same as when using the ST-compatible serial port, where

characters with parity errors are discarded.

3.	Setting the flow-control modes of some ports does not work.

4.	The Iorec(0) call does not work correctly on the Mega STe.

The program SERPTCH2.PRG fixes all of these problems.  It is a TSR and can

be run at any time, but is best run from the AUTO folder.  Newer versions

of Mega STe TOS (newer than 2.05), and TT TOS (newer than 3.05) have these

fixes built in, and thus do not need the patch.  The program analyzes what

machine it is running on, what version of TOS is running, and whether it

has already been installed, and reports what it finds.  It prints

"SERPTCH2 serial patch now installed" when it installs itself, or a message

explaining that it is not needed and why.  It uses an imperfect method

(XBRA) to see if it has already been installed, but no harm will come if it

is accidentally installed twice.

The program ("SERPTCH2.PRG") and this document file ("SERPTCH.DOC") are

placed in the public domain.  You may copy them, give them to friends,

upload them to bulletin boards, etc.  We ask that you not modify the code

or documentation, or that if you do, you name it something other than

SERPTCH.